| Group: | SP 1ST 20 |
| --- | --- |
| Type: | CONST PLANS - STORMWATER |
| Status: | REVISIONS REQUIRED |
| Date Submitted: | 6/30/2021 |
| Date Due: | 7/29/2021 |
| Date Completed: | 7/28/2021 |
| Reviewer: | Michael Walton |
| Notes: | 7/28/2021 1:32:22 PM 1. All storm sewer systems collecting drainage from the proposed drive aisles and parking areas shall be considered public and must adhere to the standards and specifications outlined within Section 6.3.3 of the Town’s SDECM, including the use of RCP and dedication of drainage easements. Specifically, there appears to be a trench drain and storm sewer proposed at the car wash entrance to route stormwater away from the car wash water. This system is collecting drainage from the parking lot and must meet all the criteria of section 6.3.3. I AM FIXING THIS. I TALKED WITH MIKE WALTON ABOUT THIS ON 8-9-2021. 2. Provide a typical section for the swale proposed south of the car wash. NEED. 3. Provide a plan and profile for all storm sewer infrastructure proposed for Lot 2 (ie. the system south of the car wash). I TALKED TO MIKE WALTON ON 8-9-2021. THIS WILL NOT BE RCP. NO PROFILES NEEDED. 4. The proposed Type R inlet’s plan and profile is currently shown on both the SP19-063 and SP19-064 construction plan sets. Determine which lot’s proposed design will encompass this work and remove from the opposite plan set THIS IS PART OF C-STORE WORK. SEE MY COMMENTS ON THE OTHER STORM EASEMENT MEMO. |