Group:; SP 1ST 20. Type:; CONST PLANS - ENVIRONMENTAL. Status:; REVISIONS REQUIRED. Date Submitted:; 6/30/2021. Date Due:; 7/29/2021. Date Completed:; 7/28/2021. Reviewer:; Michael Walton. Notes:; 7/28/2021 1:30:57 PM GENERAL COMMENTS 1. Provide a CBMP plan specific to the work proposed in site plan application SP21-064 (ie exclude the control measures and disturbance extents specific to work proposed for site plan application SP21-063). NEED TO DO THIS. NOTICE I ADDED SEPARATE ROCK ENTRANCES, PORTAPOTTY, DUMPSTER, CONC WASH, PARKING JUST FOR CAR WASH. 2. Omit all existing and proposed utilities from the CBMP Drawings, except those relating to stormwater. COMPLETE. 3. Label and identify the ratio of all slopes that are 4:1 or greater. COMPLETE. LOCATED WEST OF CAR WASH AND ALONG SOUTH WALL OF CAR WASH. Please note Erosion Control Blanket (ECB) is required on all slopes steeper than 4:1. COMPLETE. 4. Replace Curb Socks (CS) with Rock Socks (RS) to match Town standard nomenclature and details. COMPLETE. 5. Include within the plan set all of the town’s 31 CBMP Notes & Details. NEED TO DO THIS. 6. Provide unique blocks for every inlet protection type used. This helps avoid confusion in the field regarding which control measure is being specified. WE SHOULD HAVE ALL THE ANTICIPATED PROTECTIONS SHOWN. 7. Add the following note to the initial and final CBMP plan sheets: “LOT PROTECTION (LP) IS REQUIRED ON COMMERCIAL LOTS WHEN LANDSCAPING IS NOT POSSIBLE.” ADDED TO NOTES. INITIAL CBMP PLANS 8. Provide and identify Inlet Protection for Area Inlet Not in Pavement (IPAN) for the northern most existing inlet as well. COMPLETE 9. Please consider switching the location of the Stabilized Staging Area (SSA) and Stockpile (SP). Connecting the SSA to the Vehicle Tracking Control (VTC) greatly limits sediment tracking by allowing for work vehicles to enter and exit the site without needing to cross actively disturbed areas. GOOD IDEA. COMPLETE. 10. Provide a callout outside the perimeter controls along the public sidewalk(s) to be modified/replaced with the project that states the following: “ALL WORK WITHIN PUBLIC RIGHT-OF-WAY (ROW) WILL REQUIRE A TOWN ROW PERMIT. INITIAL AND INTERIM CBMPS FOR WORK WITHIN THE TOWN’S ROW SHALL BE COORDINATED WITH THE TOWN’S ENVIRONMENTAL INSPECTOR PRIOR TO THE BEGINNING OF ANY ROW WORK.” COMPLETE. INTERIM/FINAL CBMP PLANS 11. Provide and identify sediment control logs (SCL) along all paved areas adjacent landscape/pervious areas. This includes landscape islands and all pedestrian walks. COMPLETE..