6901 South Pierce Street, Suite 315 Littleton, Colorado 80128     Voice 303-232-8088

1March 01, 2018

Alex Mestdagh, P.E.

Town of Parker

Engineering Department

20120 East Mainstreet

Parker, CO 80138

Re:	Parker Pointe Minor Development Plat – Engineering 1st Review

Parker Pointe Lot 9 Retail Building – Engineering 1st Review Parker

Pointe Lot 10 Retail Building – Engineering 1st Review

Dear Mr. Mestdagh,

Attached please find revised construction drawings reflecting comments addressed from your review letter dated December 29, 2017. I have reproduced the comments below with responses added in red.

General Comments

- The public improvements associated with this project will be required to be secured via a Subdivision Agreement. On future submittals, please include a cost estimate for all Public Improvements. I would like to get the next review comments to prepare an accurate estimate.

- Prior to any approvals, the applicant will need to provide the executed access easement for the offsite access on Stroh Road. The applicant will also need to obtain a drainage easement dedicated to the Town for the pond if it is to remain in its proposed location. Comment noted and agreed.

- On future submittals, the documents associated with the subdivision file (SUB17-058) should only include the site’s major infrastructure; specifically work on Parker Road, Stroh Road, the private internal roads, the detention pond, and the site’s major utility infrastructure. The documents associated with the site plans for Lots 7 and 8 (SP17-095 and SP17-096) should include CD’s, a site plan, and a traffic conformance letter. Coordinate with Town staff prior to resubmittal. The CDs submitted now only show infrastructure work. Due to the extensive site plan comments and negotiations, CDs for Lots 10 and 11 will follow upon confirmation that we have an acceptable site plan.

Plat

- Add the following notes to the plat:

ACCESS EASMENT ARE HEREBY GRANTED OVER ALL PRIVATE ENTRANCES AND INTERNAL ROADWAYS WITHIN THE DEVLOPMENT FOR THE BENEFIT OF ALL CURRENT AND FUTURE OWNERS OF THESE PROPERTIES FOR INGRESS, EGRESS AND TRAFFIC CIRCULATION. SHOULD PROPERTIES BE FURTHER SUBDIVIDED INTO ADDITIONAL LOTS, ALL SUCH LOTS SHALL HAVE THE RIGHT TO USE ALL SUCH ENTRANCES AND ROADWAYS FOR PUBLIC ACCESS PURPOSES. Note added.

Within the sight triangles, as shown, limited landscaping shall be allowed but no solid structures or trees will be permitted. Solid structures shall include, but not be limited to, fences, mailboxes, and utility boxes. Landscaping within the sight triangles will be limited to shrubs and plantings that at maturity will be no taller than two feet. Landscaping within the sight triangle shall be maintained by the property owner or appropriate association. Noted and addressed.

- Show sight triangles on the plat. Triangles added.

- Dedicate 25’ drainage easements over all storm sewer. Drainage easements added.

Traffic Study

Please see traffic study comments on eTrakit. No comments were found.

Construction Plan

- Add the Town’s sight distance certification to the cover sheet. This can be found in Section 3 of the RDCCM. Certification added.

- Use the Town’s updated signature block.  This has been uploaded to eTrakit. Block updated.

- Update the Town of Parker contact phone number to (303) 841-9546. Comment addressed.

- Ensure that through lanes align with the existing subdivision entrance across Stroh Road at the eastern access. Lanes are shifted to align.

- Revise the curb return radii at the east access to be 30 feet. Comment addressed.

- Please revise the spacing between the west access on Stroh Road and intersection of the private drives. The east-west private access road should be shifted to straddle the Lot 1- Lot 2 property line to provide adequate intersection sight distance from vehicles entering from Stroh Road. Access drive is shifted south.

- Remove all “future access” callouts on Lot 12. Access on that lot cannot be approved until the proposed use, and associated traffic implications, are evaluated. This will be completed with the site plan application for Lot 12. Comment addressed.

- In order to avoid access spacing issues along the private drive, it may be necessary to have shared accesses between Lots 9 and 8A and/or Lots 8 and 8A. Please make any necessary changes to the CD’s and dedicate appropriate access easements. The access to lot 9 is revised. Blanket cross access easement to be included in the CC&R’s for the project.

- Show slopes greater than 4:1 on the grading plan. Comment addressed. Only area greater than 4:1 is along Parker Road near the box culvert.

- Add cross sections of the proposed roadway improvements for both the Parker and Stroh Road plans. Comment addressed.

- Show signal pole locations and callout any necessary modifications. Signal pole shown in new island. Signal modification plans will be required to adjust signal heads.

- Add the following note to the overall utility plan: Note added.

The proposed utility connection will require a Town right-of-way permit prior to commencing work. The Town prefers connections to be bored to the extent possible, and any street cut allowed by the Town will be required to be patched according to the Town's construction standards and details. The limits of patching will be determined by the Town at the time of construction.  The Town of Parker does not allow roadway closures for utility work.

- Add the following note to the Parker Road plan and profile: Note added.

All work on Parker Road will require CDOT approval and a CDOT right-of-way permit.

- Add the following note to the Stroh Road plan and profile: Note added.

All work on Stroh Road will require a Town right-of-way permit. Any work on Stroh Road that influences traffic on Parker Road will also require a CDOT right-of-way permit.

- Please clarify with Town staff why the detention pond’s outfall is routed back through the site and outlets to near the culvert under Parker Road. The Town would prefer that the pond outfall to the nearest feasible point in the channel. The stream is not deep enough opposite the pond to outfall. It must be routed west to a lower elevation.

- Grade lots fronting Parker Road back into site or clearly show the intent for them to drain to pond. Also, in the interim, provide temporary sediment basins for the lots draining towards Parker Road. The lots must be graded as shown due to cover limitations over the Magellan pipeline and additionally to provide sight lines to the rear lots. The storm sewer is placed as deep as possibly to facilitate the ability to buck grade and drain to the pond.

- On future submittals, provide a pond layout sheet. A pond detail sheet is added.

- The CD’s and drainage report will need to address how flows from the Stroh Road widening will be routed to the storm sewer network. Storm sewer is extended to Stroh Road to pick up flows. Drainage calculations and discussion are revised.

- The southern-most lane on Stroh Road should extend past the RIRO entrance to the development to provide adequate merging distance for vehicles continuing east. Lane alignment is revised as requested.

- Show the existing lane configuration of Stroh Road across the Parker Road intersection and ensure that thru lanes align across the intersection. Additional pavement may be necessary at the intersection to align the lanes. Lanes shown and additional pavement added.

- Please coordinate with Town and Douglas County staff regarding the location of the detention pond and the proposed offsite grading. There are several concerns regarding having the pond serving this property located outside of Town limits. Comment noted.

- The triangular forebay shown in the pond does not comply with Town criteria. Please revise per Figure 7.1 in the Storm Drainage and Environmental Criteria Manual. The forebay geometry is revised to more closely represent the Town standard. It is stretched out to meet UDFCD volume requirements.

- The Town does not have criteria regarding baffle blocks or energy dissipation in the forebay. It is the responsibility of the design engineer to determine the need for baffles and provide a detail if they are deemed necessary. Baffle blocks are removed. They may cause additional turbulence in a smaller storm and re-suspend particulate matter.

- Show 5 and 100 – year HGL’s on the storm sewer profiles. HGL’s added.

- Please show elevations on the storm profile grid on sheet C4.11. Comment addressed.

- Show all utility crossings on the storm sewer profiles. We will add all crossings upon firming up alignments with Parker Water and San.

- Provide profiles for all storm sewer; including laterals. Additional profiles added.

- Show pond details on a specific scale. Scaled pond details are added.

- Revise the pond outlet structure detail per Figure 7.5 in the SDECM. Structure revised.

- Provide an outlet structure plan view per Figure 7.5. Detail added.

- Add a note to Sheet C4.93 stating that fibermesh concrete is to be used in the trickle channel. Notation added on sheet 4.15.

- On future submittals, provide a forebay detail. Forebay detail added.

- The 100-year restrictor plate should not have a circular hole. The plate should be positioned above the outlet pipe invert at the height specified in the UD Pond Calculation Sheet to maintain the 100-year flow.  Please update the UD Pond calc sheet accordingly. A circular hole is used per the UDFCD spreadsheet.

- The manhole proposed at Design Point 23 should be replaced with an area inlet to collect the flows from the swale that runs along the east side of the property. Revisions to the storm sewer have moved this manhole so it would no longer pick up swale runoff.

# Stormwater Review Comments

The following comments concern drainage, erosion and sediment control, and non-point source pollution control issues for the subject property. They are based upon our review of the submittal documents against the criteria presented in the Town of Parker’s, Storm Drainage and Environmental Criteria Manual (SDECM), as revised, February, 2014. Additional regulatory and planning documents were utilized in the review, and are referenced in the comments where appropriate.

Final Drainage Report

- Several of the storm pipes are shown with an extremely low slope. Please ensure that all storm sewer has a minimum velocity of 3 ft/s to ensure the pipes are self-scouring. Modeling indicates a couple pipes slightly below 3ft/sec, but higher than 2 ft/sec.

- All flows generated from this development must comply with water quality and detention requirements. Please make necessary grading and storm sewer revisions to route all private drive runoff to the detention pond. Please also delineate any basins that release flows undetained from the site and adjust the pond calculations per the compensatory storage requirements outlined in 7.2.4 in the SDECM. Note that runoff associated with large sections of the private drives do not qualify for compensatory storage and need to be routed to the detention facility. The site is revised to capture the majority of site runoff. Note that offsite paved areas from both Stroh Road and Preservation Trail are captured and treated for both detention and water quality which mitigates minor uncaptured flows.

- On future submittals, include swale calculations and ensure proper reinforcement is provided. Swale calculations added.

- It appears Basin P, the land area of the detention pond, is not delineated on the drainage plan. The pond is now included in Basin OS-1

- The table on page 6 in Section V has release times shown under the “release rate” column. Table is revised to reflect revised design.

- Basin SR1 currently drains to a roadside ditch. This ditch will be removed with the Stroh Road widening. Please provide discussion on how these flows will be accommodated. It may be necessary to revise the grading and have the swale north of the property collect these flows. Storm sewer extended to collect flows. Discussion and calculations added to report.

- It appears the pond is extremely oversized; please clarify with Town staff why this is. Pond size is reduced.

- On future submittals, include UD inlet calculations. Calcs added.

Sediment and Erosion Control

- Add the following note to each plan sheet: Note added.

To reduce the potential for clogging of debris grates, no straw mulch shall be used within the Excess Urban Runoff Volume of a detention basin. Instead, erosion control blanket shall be installed for a width of at least 6‐feet on both sides of the concrete low flow channels or up to a depth of 1‐foot in soil riprap or benched low flow channels. Additionally, hydraulically‐applied wood fiber mulch, stabilized fiber matrix, bonded fiber matrix, or flexible growth medium shall be used in all other areas within the Excess Urban Runoff Volume of the detention basin. The blanket and hydraulically‐applied mulch shall comply with the materials and installation requirements for erosion control blankets as required by the Town’s CBMP program, and where applicable, the product manufacturer.

- Add a note on all land adjacent to the project stating that no work shall occur in these areas. Note added.

- Add J-hooks to the silt fence along the north and west boundaries to limit the concentrated flows. J hooks added.

- Add a TSB to the southwest corner of the project to contain flows from the west side of the project. Several TSB’s added.

- Show all TSB sizing calculations and standpipe outfall locations. Sizing info and standpipes added.

- Show the ratio of all slopes greater than 4:1. Only areas greater than 4: 1 are adjacent to the floodplain near the box culvert under Parker Road.

- Add a note stating that lot protection is required on all commercial lots when landscaping is not possible. Note added.

- Show temporary irrigation in all areas of permanent native vegetation not already served by permanent irrigation. Ensure the temporary irrigation is shown on the landscaping plan. Comment addressed.

- Show erosion control blanket on both sides of the trickle channel in the detention pond. Added on Douglas County plan set.

- Show silt fence all the way to the proposed sanitary sewer connection west of Parker Road. Silt fence added.

- Show the limits of the RCZ and the 100-yr floodplain on the west side of Parker Road adjacent to the SS tie-in. Regions added.

- Change DCT to DTC, and MMP to MWP. Corrected.

Thank you for your comments. Please let us know if you have additional requirements and we will provide prompt resolution.

Jerry W. Davidson, P.E.

For and On Behalf of

Perception Design Group, Inc.