DESCRIPTION

A parcel of land being A PORTION OF LOTS 1 THROUGH 6, TRACT A, LINCOLN PROFESSIONAL PARK FILING NO. 1 RECORDED AT RECEPTION NO. ____________, SITUATED IN THE NORTHWEST QUARTER OF SECTION 15, TOWNSHIP 6 SOUTH, RANGE 66 WEST OF THE 6TH PRINCIPAL MERIDIAN, COUNTY OF DOUGLAS, STATE OF COLORADO, MORE PARTICULARLY DESCRIBED AS FOLLOWS.

Commencing at THE NORTHEAST CORNER OF SAID LOT 6 OF SAID LINCOLN PROFESSIONAL PARK FILING NO. 1;

thence South 81°36'12" West, a distance of 13.40 feet to the POINT OF BEGINNING;

thence South 05°49'30" East, a distance of 19.30 feet;

thence SOUTH 82°37'51" WEST, a distance of 30.00 feet to the south line extended EASTERLY OF A MULTI-USE UTILITY EASEMENT DEDICATED BY SAID LINCOLN PROFESSIONAL PARK FILING NO. 1;

thence NORTH 90°00'00" West ALONG said south line AND THE EASTERLY EXTENSION THEREOF, a distance of 266.77 feet to a point of curvature;

thence ALONG THE SOUTH LINE SAID MULTI-USE UTILITY EASEMENT THE FOLLOWING EIGHT (8) COURSES:

- along the arc of said curve to the right an arc length of 25.57 feet, said curve having a radius of 65.00 feet, a central angle of 22°32'24", and a chord which bears North 78°43'48" West a chord distance of 25.41 feet;

- North 67°27'36" West, a distance of 84.43 feet to a point of curvature;

- along the arc of said curve to the left an arc length of 6.24 feet, said curve having a radius of 15.00 feet, a central angle of 23°49'50", and a chord which bears North 79°22'31" West a chord distance of 6.19 feet;

- South 88°42'34" West, a distance of 117.66 feet to a point of curvature;

- along the arc of said curve to the left an arc length of 7.85 feet, said curve having a radius of 15.00 feet, a central angle of 30°00'08", and a chord which bears South 73°42'29" West a chord distance of 7.77 feet;

- South 58°42'25" West, a distance of 135.13 feet to a point of curvature;

- along the arc of said curve to the right an arc length of 34.11 feet, said curve having a radius of 65.00 feet, a central angle of 30°04'03", and a chord which bears South 73°44'27" West a chord distance of 33.72 feet;

- South 88°46'28" West, a distance of 59.87 feet TO THE WEST LINE OF LOT 4, SAID LINCOLN PROFESSIONAL PARK FILING NO. 1;

thence North 01°41'43" West ALONG SAID WEST LINE AND THE WEST LINE OF SAID TRACT A, a distance of 50.00 feet TO THE NORTHwest corner of sAID MULTI-USE UTILITY EASEMENT;

thence ALONG SAID NORTH LINE THE FOLLOWING fifteen (15) COURSES:

- North 88°46'28" East, a distance of 60.28 feet to a point of curvature;

- along the arc of said curve to the left an arc length of 7.87 feet, said curve having a radius of 15.00 feet, a central angle of 30°04'03", and a chord which bears North 73°44'27" East a chord distance of 7.78 feet;

- North 58°42'25" East, a distance of 135.13 feet to a point of curvature;

- along the arc of said curve to the right an arc length of 34.04 feet, said curve having a radius of 65.00 feet, a central angle of 30°00'08", and a chord which bears North 73°42'29" East a chord distance of 33.65 feet;

- North 88°42'34" East, a distance of 91.27 feet;

- North 01°17'26" West, a distance of 12.20 feet;

- North 88°42'34" East, a distance of 33.77 feet;

- South 01°17'26" East, a distance of 12.62 feet to a point of non-tangent curvature;

- along the arc of said curve to the right an arc length of 19.65 feet, said curve having a radius of 65.00 feet, a central angle of 17°19'17", and a chord which bears South 76°07'15" East a chord distance of 19.58 feet;

- South 67°27'36" East, a distance of 84.43 feet to a point of curvature;

- along the arc of said curve to the left an arc length of 5.90 feet, said curve having a radius of 15.00 feet, a central angle of 22°32'24", and a chord which bears South 78°43'48" East a chord distance of 5.86 feet;

- North 90°00'00" East, a distance of 165.97 feet;

- North 00°00'00" West, a distance of 2.75 feet;

- North 90°00'00" East, a distance of 20.00 feet;

- South 00°00'00" East, a distance of 2.75 feet;

thence North 90°00'00" East ALONG SAID NORTH LINE AND SAID LINE EXTENDED EASTERLY, a distance of 75.47 feet;

thence South 50°51'23" East, a distance of 42.70 feet to the POINT OF BEGINNING.

Said Parcel contains 38,299 square feet or 0.88 acres, more or less.

BASIS OF BEARINGS: BEARINGS ARE BASED ON THE NORTH LINE OF THE NORTHWEST QUARTER OF SECTION 15, AS BEARING SOUTH 89°29’00” WEST PER THE PLAT OF LINCOLN PROFESSIONAL PARK FILING NO. 1.

PREPARED BY:  AARON MURPHY

PLS 38162

ON BEHALF OF: HARRIS KOCHER SMITH

1120 LINCOLN STREET, SUITE 1000

DENVER, CO 80203

303.623.6300