Hess Ranch Development

Town of Parker, Colorado

PD DEVELOPMENT GUIDE - AMENDMENT

November 11, 2016

Owner:

E5X Management

7353 South Alton Way

Centennial, Colorado 80112

303.770.9111

Contact: Matt Janke/Chris Elliot

Planning Consultants:

PCS Group, Inc.

1001 16th Street, #3-B-180

Denver, Colorado 80265

720.259.8246

Contact: John Prestwich

Engineering Consultants:

CVL Consultants of Colorado, Inc.

10333 E. Dry Creek Road, Suite 240

Englewood, Colorado 80112

720.482.9526

Contact: Karl Knapp/Jim Jannicke

Traffic Consulting

David Evans & Associates, Inc.

1331171" Street, Suite 900

Denver, 720.946.0969.296.4300

Contact Anna Ericson

June 26, 2015

I.	GENERAL PROVISIONS	3

A.	Application	3

B.	Density Standards	3

C.	Density Variation	3

D.	Planning Area Boundaries	3

E. 	Underground Utility Requirements	4

F. 	Homeowner Associations	4

II. 	DEFINITIONS	4

C. 	Definitions:	4

III. 	LAND USE REGULATIONS	8

A. 	General	8

B. 	Residential (SFd)	9

C. 	Residential (SFe)	11

D. 	Residential (SFf)	14

E. 	Residential (SFg)	16

F. 	Residential (THd)	18

G. 	Residential (THe)	20

H. 	Mixed Use (MU)	22

I. 	Parks	27

J.	Open Space (OS)	30

K. 	Trails	30

IV. 	ADDITIONAL PLANNING AREA REQUIREMENTS	31

A. 	General	31

B. 	Lone Tree Intergovernmental Agreement	31

C.	 Douglas County Intergovernmental Agreement	32

V. 	ENFORCEMENT AND ADMINISTRATION	32

A. 	Incorporation of Planned Development Master Plan	32

B. 	Effect of Recorded Plan	32

# I.	GENERAL PROVISIONS

## A.	Application

The standards outlined in the Development Guide shall· apply to all property contained within the "HESS RANCH DEVELOPMENT PLAN AND GUIDE" (the "Development Plan") except as provided herein. The standards outlined in the Development Plan may be divergent from the zoning regulations contained in Chapter 13.04 of the Parker Municipal Code, but not any other Chapter of the Parker Municipal Code. In the event there is a conflict between the standards contained in the Development Plan and the Parker Municipal Code (excluding Chapter 13.04) then the Parker Municipal Code shall control. The terms "DEVELOPMENT PLAN" and "MASTER PLAN" may be used interchangeably.

This Development Guide is only one of several documents that will help guide the Hess Ranch Planned Development. The Development Guide and the Development Plan are the planning documents for this development.

The Development Agreement is a third document that will help frame the overall development with specific language of agreements, triggers, responsibilities, etc. Please refer to this document to gain a full understanding of the Hess Ranch Planned Development.

## B.	Density Standards

The Dwelling Unit Density permitted in any residential Planning Area is an Average Density that shall apply to the entire residential Planning Area and shall not be specifically applicable to any portion thereof. The Average Density of any residential Planning Area shall be computed by dividing the total number of Dwelling Units ln the residential Planning Area by the gross acres in the residential Planning Area.

## C.	Density Variation

In no event shall the total number of 3,379 residential Dwelling Units within all Planning Areas as set forth on the "HESS RANCH DEVELOPMENT PLAN AND GUIDE" be exceeded.

In general, residential Planning Areas may contain up to an additional twenty percent (20%) of the number of units allowed for each Planning Area, as set forth in the ''HESS RANCH PD DEVELOPMENT PLAN AND GUIDE'', as determined by the Developer. The total number of Dwelling Units actually developed in a Planning Area may be less than the number established on the Development Plan. Unbuilt or unused dwelling units resulting from the Final Plat of any Planning Area, may be transferred to other areas provided that the receiving Planning Area not exceed 20% of the original density provided for on the Hess Ranch Development Plan. The transfer of any unbuilt or unused dwelling units may occur at any time throughout the development process of Hess Ranch. Any increase or decrease of dwelling units within a Planning Area by more than 20% must be processed as a Major Amendment under Section 13.04.240 of the Parker Municipal Code.

## D.	Planning Area Boundaries

Wherever a Planning Area abuts a street as shown on the "HESS RANCH DEVELOPMENT PLAN", the Planning Area boundary is the edge of the abutting right-of-way of such street (except Crowfoot Valley Road). Wherever a Planning Area does not so abut a street, the Planning Area boundary shall be as shown on the "HESS RANCH DEVELOPMENT PLAN". Modifications in Planning Area boundaries and streets may be accomplished by the Developer by final road alignments or engineering refinements shown on a Site Plan or Plat, without any amendment to the "HESS RANCH DEVELOPMENT PLAN" provided the Planning Area does not increase or decrease by more than twenty percent (20%) in size.

## E. 	Underground Utility Requirements

All electrical and communications distribution lines shall be placed underground. All transmission lines shall be underground unless same cannot be accomplished by direct burial.

## F. 	Homeowner Associations

Homeowner Associations composed of property owners in residential areas may be created for the following purposes: a) to provide for the continued development, improvement and maintenance of properties and facilities which it owns or administers, and b) to protect the investment, enhance the value, and control the use of property owned by its members.

Homeowner's Associations or special districts may be created in residential areas where common lands or facilities are to be owned and maintained by the Homeowner's Association or special district.

# II. 	DEFINITIONS

A.	Purposes:  It is the purpose of this Article to define words, terms and phrases contained within 			this PD Development Guide. See the Town of Parker Land Development Code for other 				definitions not defined herein.

B.	Word Usage: In the interpretation of this Guide, the provisions and rules of this section shall 			be observed and applied, except when the context requires otherwise.

1. 	The particular controls the general.

2. 	In the case of any difference of meaning or implication between the text of this 			Guide and any caption or table, the text shall control.

3. 	Words used or defined in one tense or form shall include other tenses and 			derivative forms.

4. 	Words in the singular number shall include the plural number and words in the 			plural number shall include the singular number.

5. 	The masculine gender shall include the feminine and the feminine gender shall 			include the masculine.

6. 	The word "shall" is mandatory.

7. 	The word "may'' is permissive.

## C. 	Definitions:

1. 	Abut. To have a common border with.

2. 	Accessory Buildings. Detached subordinate building(s) or Structure(s), the use 			of which is customarily incidental to that of the Principal Building or to the main 			use of the land and which is located on the same lot with the Main Building or 			use, including carriage house units located above detached and semi-detached 			garages, designated for and occupied by not more than one family, and with a 			maximum size of 600 square feet.

3.	Age Targeted Dwelling. Housing designed to appeal to the lifestyle needs of 			buyers over 55 years of age.

4. 	Age Restricted Dwelling. Housing limited to buyers and occupants 55 years of 			age or older.

5. 	Building, Principal or Main. A building or buildings which may contain one or 			more Dwelling Unit(s) in which is conducted one or more of the permitted 			Principal Uses of the Lot or project in which it is situated and including areas 			such as garages, carports, storage sheds which are attached to or architecturally 		integrated with the principal building.

6. 	Building Front.  That exterior wall of a Building which faces a Front Lot Line of a 			Lot.

7. 	Building Ground Coverage. The amount of land covered or permitted to be 			covered by a Building.

8. 	Building Setback.  The distance between the Building and the adjacent street 			right-of-way, property line, exterior of the Private Street Easement, or external 			planning area boundary.

9. 	Building Side.  That exterior wall of a Building which faces a Side Lot Line of a 			Lot.

10.	Building Rear.  That exterior wall of a Building which faces a Rear Lot Line of a 			Lot.

11. 	Car Care Center.  An establishment providing sales of vehicle fuel and such 			services as lubrication, oil and tire changes, and minor repairs. This use does not 		Include paint spraying or body repair.

12. 	Density, Gross or Average.  A ratio of number of Dwelling Units per acre 			calculated by dividing the number of Dwelling Units within the boundary of one 			or more residential Planning Areas or portion thereof by the number of acres 			contained within the Planning Area(s) or portions thereof; including all land 			within said Planning Area(s) or portion thereof reserved or dedicated for Parks, 			Open Space, recreation, educational, community, or public use, and Public 			Street rights-of way and Easements. For calculation purposes, the gross density 			excludes the 100-year floodplain as mapped by FEMA and slopes of 20% or 			greater.

13. 	Density, Net.  A ratio of number of Dwelling units per acre calculated by dividing 		the number of Dwelling Units within the boundary of one or more residential 			Planning Areas or portion thereof by the number of acres contained within the 			Planning Area(s) or portion thereof; excluding all land within said Planning 			Area(s) or portion thereof reserved or dedicated for Roads, Parks, Open Space, 			recreations, educational, community or public use, and Public Street rights-of-			ways and easements. For calculation purposes, the gross density excludes the 			100-year floodplain as mapped by FEMA and slopes of 20% or greater

14. 	Dwelling Unit, Atrium. An alternative type of Single Family Detached or 				Attached Dwelling Unit in which the private Individual Open Space associated 			with each Dwelling Unit is consolidated into a private yard surrounded by 			buildings or walls forming an atria. This atria replaces the conventional front 			side and rear yards associated with traditional Single Family Detached Dwelling 			Units.

15. 	Dwelling Unit, Clustered Single Family.  A type of Single Family Detached or 			Attached Dwelling Unit development that combines reduced lot size with 			compensating amounts of Open Space within the proposed development.

16. 	Dwelling Unit, Patio.  An alternative type of Single Family Detached or Attached 			Dwelling Unit in which the private Individual Open Space associated with each 			Dwelling Unit is consolidated into a garden or patio area commonly located 			toward the side or rear of the Lot.  This patio area replaces the conventional 			front, side and rear yards associated with traditional Single Family Detached 			Dwelling Units.

17. 	Dwelling Unit, Single Family Attached.  A Building designated for and 				containing more than one Dwelling Unit and occupied by not more than 6 			families within one Structure each  which is separated from the others by one 			(1) or more un-pierced walls from ground to roof.

18. 	Dwelling Unit. Single Family Detached.  A type of Dwelling Unit designated for 			and occupied by not more than one family and having no roof, wall or floor in 			common with any other Dwelling Unit.

19. 	Dwelling Unit, Townhome.  A type of Single Family Attached or Multi-Family 			Dwelling Unit in which each single Dwelling Unit goes from ground to roof and is 		separated from other Townhome Dwelling Units by a common wall. Each 			Townhome Dwelling Unit has Individual outside access. Townhome Dwelling 			Units may have no side yards, or front and rear yards. May include duplex, 			triplex, fourplex, or more.

20. 	Dwelling Unit, Zero Lot Line.  An alternative type of Single Family Detached 			Dwelling Unit which is situated on a Lot so that one or more sides rest directly 			on the boundary line of the Lot.

21. 	Dwelling Unit, Live/Work.  An alternative type of Townhome Dwelling Unit in 			which a range of retail goods and services, consistent with the Mixed Use (MU) 			designation are permitted, in combination with residential use, and where 			residential setbacks apply.

22. 	Dwelling Unit, Condominium.  A type of Single Family Dwelling Unit.  The unit is 			individually owned, each owner receives a recordable deed to the individual 			unit purchased, including the right to sell, mortgage, etc., that unit and sharing 			in joint ownership of any common grounds, passageways, etc.

23. 	Easement, Private Street.  An acquired right of use, interest or privilege in land 			owned by another containing a privately owned access way constructed to the 			specifications of the Town of Parker but not maintained by the Town of Parker. 			Town of Parker and Parker Fire Protection District approved fire access shall be 			provided in all cases.

24. 	Exterior Storage.  Outdoor storage of fuel, raw materials, products and 				equipment. In the case of lumberyards, exterior storage includes all impervious 			materials stored outdoors.

25. 	Garage, Parallel.  A garage, attached or detached, which is oriented so that the 			axis which corresponds with the garage opening is substantially parallel to the 			adjacent street from which the Lot derives access.

26. 	Garage, Perpendicular. A garage, attached or detached, which is oriented so 			that the axis which corresponds with the garage opening is substantially 			perpendicular to the adjacent street from which the Lot derives access.

27. 	Group Care Facilities. A facility where three or more persons are provided 			supervision and a planned treatment of counselling, therapy or other 				rehabilitative social services in a family environment. Such residence must be 			licensed by or operated by a federal, state, county or local judicial 				health/welfare agency or serve only referrals from such agencies. Examples of 			group care facilities are: group foster care, children's homes, halfway houses, 			rehabilitative centers, assisted living, maternity or senior citizen homes.

28. 	Land Use Category.  A set of permitted land use types which are aggregated to 			form a land use 	classification similar in nature to zoning districts. Each Land Use 			Category is identified on the PD Master Plan and in Section III. Land Use 				Regulations of this PD Development Guide.

A. 	UNIFORM LAND USES

1) 	Single Family Detached Dwellings

2) 	Duplex

3) 	Single Family Attached Dwellings-these will include atrium 					clustered patio, live work, townhome, duplex, including duplex, 					triplex and fourplex dwelling units.

4) 	Multifamily

5)	Live/Work Dwelling Units - shall be associated with all MU 					planning areas and shall be located above retail/commercial 					areas that are compatible with a live work community.  Usually 					associated with commercial developments.

6) 	Accessory Dwelling Units - shall include all structures described 					in the Parker Land Use Ordinance 13.04.170. May also include 					utility facility enclosures and well sites.

7)	Places of Religious Assembly

8) 	Accessory Structure and Uses

9)	Open Space

10)	Parks

11) 	Neighborhood Recreational Centers and Facilities

12) 	Group Home - refer to Group Care Facilities definition

13) 	Assisted Living Residence - a housing facility that provides 					assistance to disabled individuals. These individuals cannot 					perform all of the ADL's (activities of daily living) i.e. eating, 					bathing, feeding, etc. These facilitates do not provide 24 hour 					care.

14) 	Long Term Care Facility/Nursing Home

29. 	Nursery.   An enterprise which conducts the retail and wholesale sale of plants 			grown on the site, as well as accessory items (but not farm implements) directly 			related to their care and maintenance.  The accessory items normally sold are 			clay pots, polling soil, fertilizers, insecticides, hanging baskets, rakes and 			shovels.

30. 	Planning Areas. Areas of land delineated on the PD Master Plan identified with 			a phrase or symbol which designates a specific set of permitted land use types 			according to the Land Use Categories provided in Section III. - Land Use 				Regulations of this PD Development Guide. Planning Area boundaries are as 			depicted on the PD Master Plan.

31. 	Planning Commission. The Planning Commission of the Town of Parker.

32. 	Principal Use. The specific primary purpose, activity or use for which land or any 		building thereon is designed, arranged or intended, or for which it is occupied or 		maintained.

33. 	Recreational Vehicle. A vehicle or a unit that is mounted on or drawn by 			another vehicle 	primarily designed for recreation or temporary living. 				Recreational vehicles include travel trailers, camping trailers, truck campers, 			mobile homes, boats, snowmobiles and other similar recreation equipment.

34. 	Restaurant, Conventional.  An establishment whose principal business is the 			sale of food and/or beverages to customers whose principal method of 				operation includes one or both of the following characteristics: (1) customers, 			normally provided with an individual menu, are served their foods and 				beverages by a restaurant employee at the same table or counter at which food 			and beverages are consumed.  And (2) cafeteria type operation where food and 			beverages generally are consumed within the restaurant building.

35. 	Restaurant, Fast Food.  An establishment whose principal business is the sale of 		food and/or beverages in a "ready to consume" state: (1) within the restaurant 			building; (2) within a motor vehicle parked on the premises; or (3) off the 			premises as carry out orders and whose principal method of operation includes 			the following characteristics: food and/or beverages are usually served in edible 			containers or in paper, plastic or other disposable containers.

36. 	Restaurant, Fast Food With Drive Through Facilities. Drive-in or drive-through 			means an accessory use to a building or facility that by design facilitates, 			encourages or allows customers to receive services or obtain goods whole 			remaining in their motor vehicle, excluding reserved parking spaces for take-out 			goods where there is no menu board or communication facility.

37.	Setback.  The distance between the building line and the adjacent street right-			of-way line or Lot Boundary.

38.	Structure.  Anything constructed or erected, which requires permanent location 		on the ground or is attached to something having a permanent location on the 			ground, but not including fences or walls less than six (6) feet, poles, lines, 			cables, or other transmission or distribution facilities or public utilities.

39. 	Town Council.  The Town Council tor the Town of Parker.

40. 	Traditional Neighborhood Design (TND). Traditional Neighborhood Design 			standards are intended to provide flexibility allowing land use patterns 				representative of those that were planned earlier in the 20th century. 				Traditional neighborhoods generally have a broad mix of uses including Single 			Family and Multi-Family land use patterns that help to create more pedestrian 			friendly streetscapes, in neighborhoods where homes and other uses orient 			toward the street.

41. 	Transit Oriented Design (TOD). Transit Oriented Design neighborhoods contain 			a broad mix of uses including Single Family and Multi-Family residential 				combined with commercial and retail opportunities in land use patterns that 			help to create a pedestrian friendly environment, with convenient access to 			public transit.

# III. 	LAND USE REGULATIONS

## A. 	General

At the time of the Town Council's approval of this Development Plan and Guide, Planning Areas designated as SFe, SFf and SFg are within an area that is regulated by an Intergovernmental Agreement (IGA) between the Town of Parker and the City of Lone Tree. The number of dwelling units prescribed by the IGA is limited to 100 units for these Planning Areas. On September 5, 2015 the IGA expires.  The day after, September 8, 2015, the cumulative number of dwelling units allowed tor Planning Areas 27, 28, 29, 30, 31, 32 and 33 will be increased to a number greater than 100. This increase will not require a Major Amendment to the Development Plan and will be counted as part of the overall 3,379 dwelling units allowed by the Plan.

## B. 	Residential (SFd)

1. 	Intent

Provide for residential Development allowing for a variety of Single Family 			Detached Dwelling Units, Zero Lot Line Dwelling Units, Single Family Attached, 			Semi-detached, Clustered Single Family Dwelling Units, Duplex, Triplex, and 			Fourplex Dwelling Units, Townhome Dwelling Units, Condominium Dwelling 			Units and Accessory Uses.

2. 	Uses Permitted by Right

a. 	Single Family Detached Dwelling Units.

b. 	Zero Lot Line Dwelling Units

c. 	Duplex, Triplex, and Fourplex Dwelling Units.

d. 	Townhome Dwelling Units including Single Family Attached, Semi-				detached Dwelling Units and Live/Work Dwelling Units.

e. 	Condominiums

f. 	Age targeted housing (Not depicted on plan for this PD Submittal – Will 				be included in Site Plan/Subdivision Submittals)

g. 	Age restricted senior housing. (Not depicted on plan for this PD 					Submittal – Will be included In Site Plan/Subdivision Submittals)

h. 	Group Homes/Nursing Homes

i. 	Clustered Single Family Dwelling Units, including but not limited to Zero 				Lot Line Dwelling Units, Patio Dwelling Units, Two, Three, and Four 				Family Dwelling Units and Atrium Dwelling Units.

j.	A temporary sales and marketing center function developed to 					showcase a variety of builders and housing types within a limited area.

k. 	Attached or detached private garages (3 cars maximum).

I. 	Commonly associated Accessory Uses, incidental to the Principal Use or 				Building, including but not limited to: carriage house units not for sale, 				storage sheds, private greenhouses, home occupations, radio or TV 				antenna, TV satellite dishes, private tennis courts, private swimming 				pools, and private facilities for the keeping of household pets according 				to the regulations and standards within the Parker Municipal Code.

m. 	Public Open Space.

n. 	Private recreational and park uses, country clubs, recreational facilities, 				including but not limited to: tennis courts, swimming pools and jogging, 				riding, hiking and biking trails.

o. 	Public parks, playgrounds and other recreational areas.

p. 	Places of religious assembly, religious schools.

q. 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as permitted by Section 				13.04.220 of the Parker Municipal Code.

r. 	Public and private schools and related administrative facilities for 				elementary and secondary education.

3. 	Uses Permitted by Special Review

a. 	Buildings, garages and utility stations related to emergency services, 				such as ambulance, fire, police and rescue.

b. 	Neighborhood public service, health and education facilities, such as 				community centers, libraries and museums.

c. 	Child Care Centers.

4.	Accessory uses and building - Refer to Town of Parker Land Use Ordinance 			13.04.170

a. 	Shall also include utility facilities and well sites.

5.	 Development Standards

a. 	Average Density. The Average Density shall be up to four and five-				tenths (4.5) Dwelling Units per acre for each SFd Planning Area. These 				Average Densities are set forth on the PD Development Plan, and reflect 			the number of units over the entire acreage for the Planning Area. 				Densities within portions of the Planning Areas will vary depending on 				product type.

b. 	Maximum Number of Dwelling Units. The maximum number of 				Dwelling Units permitted within each SFd residential Planning Area shall 			not exceed 120% of the units shown on the PD Development Plan for 				each individual Planning Area.

c.	Building Setback. The minimum Building Setbacks in SFd residential 				Planning Area will vary depending upon the product type in any given 				portion of the Planning Area. The following setbacks will be associated 				with the different product types defined as a Use Permitted by Right.

Single Family Detached Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Zero Lot Line Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Duplex Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Townhome Dwelling Unit (Including Live/Work Townhomes and 				Condominiums)

Building Front 	4 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	4 feet from corner street

Building Sides	0 feet from other property lines & lots with common 						walls

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Patio Dwelling Unit

Building Front 	8 feet (Principal Building)

10 feet (Garage perpendicular)

6 feet (Garage parallel)

Building Sides	8 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

d. 	Building Separation. The minimum building separation shall be the 				greater of:

(i)	Seven and ½ (7.5) feet for Single Family Detached residential 					Buildings, fifteen (15) feet for Buildings containing Single Family 					Attached Dwelling Units and twenty (20) feet for other Principal 					Buildings located within SFd Land Use Planning Areas, or

ii) 	shall be governed by the applicable building code.

e. 	Building Height. No residential Buildings within SFd Planning Areas shall 				exceed thirty-five (35) feet in height.

f. 	Minimum Lot Sizes. The minimum lot sizes in SFd residential Planning 				Areas shall be thirty five hundred (3500) square feet.

## C. 	Residential (SFe)

1. 	Intent

Provide for residential Development allowing for a variety of Single Family 			Detached Dwelling Units, Zero Lot Line Dwelling Units, Single Family Attached, 			Semi-detached, Clustered Single Family Dwelling Units, Duplex Dwelling Units, 			Triplex and Fourplex Dwelling Units, Townhome Dwelling Units, Condominium 			Dwelling Units and Accessory Uses.

2. 	Uses Permitted by Right

a.	Single Family Detached Dwelling Units.

b. 	Zero Lot Line Dwelling Units

c. 	Duplex, Triplex, and Fourplex Dwelling Units.

d. 	Townhome Dwelling Units including Single Family Attached, Semi-				detached Dwelling Units and Live/Work Dwelling Units.

e. 	Condominiums

f. 	Age targeted housing (Not depicted on plan for this PD Submittal - Will 				be included in Site Plan/Subdivision Submittals)

g. 	Age restricted senior housing (Not depicted on plan for this PD 					Submittal - Will be included In Site Plan/Subdivision Submittals)

h. 	Clustered Single Family Dwelling Units, including but not limited to Zero 				Lot Line Dwelling Units, Patio Dwelling Units, Two, Three, and Four 				Family Dwelling Units and Atrium Dwelling Units.

i.	Group Homes/Nursing Homes

j.	A temporary sales and marketing center function developed to 					showcase a variety of builders and housing types within a limited area.

k. 	Attached or detached private garages (3 cars maximum).

I. 	Commonly associated Accessory Uses, incidental to the Principal Use or 				Building, including but not limited to: carriage house units not for sale, 				storage sheds, private greenhouses, home occupations, radio or TV 				antenna, TV satellite dishes, private tennis courts, private swimming 				pools, and private facilities for the keeping of household pets according 				to the regulations and standards within the Parker Municipal Code.

m. 	Public Open Space.

n.	Public and private schools and related administrative facilities for 				elementary and secondary education.

o. 	Private recreational and park uses, country clubs, recreational facilities, 				including but not limited to: tennis courts, swimming pools and jogging, 				riding, hiking and biking trails.

p. 	Public parks, playgrounds and other recreational areas.

q. 	Places of religious assembly, religious schools.

r. 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as permitted by Section 				13.04.220 of the Parker Municipal Code.

3. 	Uses Permitted by Special Review

a. 	Buildings, garages and utility stations related to emergency services, 				such as ambulance, fire, police and rescue.

b. 	Neighborhood public service, health and education facilities, such as 				community centers, libraries and museums.

c. 	Child Care Centers.

4. 	Accessory uses and building - Refer to Town of Parker Land Use Ordinance 			13.04.170

a. 	Shall also include utility facilities and well sites.

5. 	Development Standards

a. 	Average Density. The Average Density shall be up to six (6.0) Dwelling 				Units per acre for each SFe Planning Area, as set forth in the PO 					Development Plan, and reflect the number of units over the entire 				acreage for the Planning Area.  Densities within portions of the Planning 			Area will vary depending on product type.

b.	Maximum Number of Dwelling Units. The maximum number of 				Dwelling Units permitted within each SFe residential Planning Area shall 				not exceed 120% of the units shown on the PD Development Plan for 				each individual Planning Area.

c. 	Building Setback: The minimum Building Setbacks in SFe residential 				Planning Area will vary depending upon the product type in any given 				portion of the Planning Area. The following setbacks will be associated 				with the different product types defined as a Use Permitted by Right.

Single Family Detached Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Zero Lot Line Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	10 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Duplex Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	4 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Townhome Dwelling Unit (Including Live/Work Townhomes and 				Condominiums)

Building Front 	4 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines & lots with common 						walls

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Patio Dwelling Unit

Building Front 	8 feet (Principal Building)

10 feet (Garage perpendicular)

6 feet (Garage parallel)

Building Sides	8 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

d. 	Building Separation. The minimum building separation shall be the 				greater of:

i)	Seven and ½ (7.5) feet for Single Family Detached Dwelling 					Units, ten (10) feet for Buildings containing Single Family 					Attached Dwelling Units, and twenty (20) feet for other 						Principal Buildings located within each SFe Lands Use Planning 					Area, or

ii) 	shall be governed by the applicable building code.

e. 	Building Height. No residential Buildings within SFe Planning Areas shall 				exceed thirty-five (35) feet in height except by special review.

f. 	Minimum Lot Sizes. The minimum lot sizes In SFe residential Planning 				Areas shall be thirty five hundred (3500) square feet.

## D. 	Residential (SFf)

1. 	Intent

Provide for residential Development allowing for a variety of Single Family 			Detached Dwelling Units, Zero Lot Line Dwelling Units, Single Family Attached, 			Semi-detached, Clustered Single Family Dwelling Units, Duplex Dwelling Units, 			Triplex and Fourplex Dwelling Units, Townhome Dwelling Units, Condominium 			Dwelling Units and Accessory Uses.

2. 	Uses Permitted by Right

a.	 Single Family Detached Dwelling Units.

b. 	Zero Lot Line Dwelling Units.

c. 	Duplex, Triplex, and Fourplex Dwelling Units.

d. 	Townhome Dwelling Units including Single Family Attached, Semi-				detached Dwelling Units and Live Work Dwelling Units.

e. 	Age targeted dwellings. (Not depicted on plan for this PD Submittal - 				Will be included in Site Plan/Subdivision Submittals)

f. 	Age restricted dwellings. (Not depicted on plan for this PD Submittal - 				Will be included in Site Plan/Subdivision Submittals)

g. 	Group Homes/Nursing Homes

h. 	Clustered Single Family Dwelling Units, including but not limited to Zero 				Lot Line Dwelling Units, Patio Dwelling Units, Two, Three, and Four 				Family Dwelling Units and Atrium Dwelling Units.

i. 	A temporary sales and marketing center function developed to 					showcase a variety of builders and housing types within a limited area.

j. 	Attached or detached private garages (3 cars maximum).

k. 	Commonly associated Accessory Uses, incidental to the Principal Use or 				Building, including but not limited to: carriage house units not for sale, 				storage sheds, private greenhouses, home occupations, radio or TV 				antennas, TV satellite dishes, private tennis courts, private swimming 				pools, and private facilities for the keeping of common household pets 				according to the regulations and standards contained within the Parker 				Municipal Code.

I. 	Public Open Space.

m. 	Public and private schools and related administrative facilities for 				elementary and secondary education.

n. 	Private recreational and park uses, country clubs, recreational facilities, 				including but not limited to: tennis courts, swimming pools and jogging, 				riding, hiking and biking trails.

o. 	Public parks, playgrounds and other recreational areas.

p. 	Places of religious assembly, religious schools.

q. 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as permitted by Section 				13.04.220 of the Parker Municipal Code.

3. 	Uses Permitted by Special Review

a. 	Buildings, garages and utility stations related to emergency services, 				such as ambulance, fire, police and rescue.

b. 	Neighborhood public service, health and education facilities, such as 				community centers, libraries and museums.

c. 	Child Care Centers.

4. 	Accessory uses and building · Refer to Town of Parker Land Use Ordinance 				13.04.170

a. 	Shall also include utility facilities and well sites.

5. 	Development Standards

a. 	Average Density. The Average Density shall be up to three point two 				(3.2) Dwelling Units per acre for each SFf Planning Area, as set forth in 				the PD Development Plan, and reflect the number of units over the 				entire acreage for the Planning Area. Densities within portions of the 				Planning Area will vary depending on product type.

b. 	Maximum Number of Dwelling Units. The maximum number of 				Dwelling Units permitted within each SFf residential Planning Area shall 				not exceed 120% of the units shown on the PD Development Plan for 				each individual Planning Area.

c. 	Building Setback. The minimum Building Setbacks in SFf residential 				Planning Areas will vary depending upon the product type in any given 				portion of the Planning Area. The following setbacks will be associated 				with the different product types defined as a Use Permitted by Right.

Single Family Detached Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Zero Lot Line Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Duplex Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Townhome Dwelling Unit (Including Live/Work Townhomes and 				Condominiums)

Building Front 	4 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	4 feet from corner street

Building Sides	0 feet from other property lines & lots with common 						walls

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Patio Dwelling Unit

Building Front 	8 feet (Principal Building)

10 feet (Garage perpendicular)

6 feet (Garage parallel)

Building Sides	8 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

d. 	Building Separation. The minimum building separation shall be the 				greater of:

i)	seven and ½ (7.5) feet for Single Family Detached Dwelling 					Units, ten (10) feet for Buildings containing Single Family 					Attached Dwelling Units, and twenty (20) feet for other 						Principal Buildings located within each SFf Lands Use Planning 					Area, or

ii) 	shall be governed by the applicable building code.

e. 	Building Height. No residential Buildings within SFf Planning Areas shall 				exceed thirty-five (35) feet in height except by special review.

f. 	Minimum Lot Sizes. The minimum lot sizes In SFf residential Planning 				Areas shall be thirty five hundred (3500) square feet.

## E. 	Residential (SFg)

1. 	Intent

Provide for residential Development allowing for Estate Single Family Detached 			Dwelling Units and Accessory Uses.

2. 	Uses Permitted by Right

a. 	Single Family Detached Dwelling Units.

b. 	Attached or detached private garages.

c. 	Commonly associated Accessory Uses, incidental to the Principal Use or 				Building, including but not limited to: carriage house units not for sale, 				storage sheds, private greenhouses, home occupations, radio or TV 				antennas, TV satellite dishes, private tennis courts, private swimming 				pools, and private facilities for the keeping of common household pets 				according to the regulations and standards contained within the Town 				of Parker Zoning Ordinance.

d. 	Public parks, playgrounds and other recreational areas.

e. 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as permitted by Section 				13.04.220 of the Parker Municipal Code.

3. 	Uses Permitted by Special Review

a. 	Neighborhood public service, health and education facilities, community 			centers, libraries and museums.

4.	 Accessory uses and building - Refer to Town of Parker Land Use Ordinance 			13.04.170

a. 	Shall also include utility facilities and well sites.

5. 	Development Standards

a. 	Average Density. The Average Density shall be up to one unit per one 				and one half acres (0.67 Dwelling Units per acre) for each SFg Planning 				Area. These Average Densities are set forth on the PD Development 				Plan, and reflect the number of units over the entire acreage for the 				Planning Area. Densities within portions of the Planning Area will vary 				depending on product type.

b. 	Maximum Number of Dwelling Units. The maximum number of 				Dwelling Units permitted within each SFg residential Planning Area shall 				not exceed 120% of the units shown on the PD Development Plan for 				each individual Planning Area.

c. 	Building Setback: The minimum Building Setbacks in SFg residential 				Planning Areas shall be:

Building front	20 feet

Building side	15 feet

Building rear	20 feet

d. 	Building Separation. The minimum building separation shall be the 				greater of:

i) 	twenty (20) feet for Single Family Detached residential Buildings 				located within SFg Lands Use Planning Areas, or

ii) 	shall be governed by the applicable building code.

e. 	Building Height. No residential Buildings within SFg Planning Areas shall 				exceed thirty-five (35) feet in height except by Special Review

f. 	Minimum Lot Sizes. The minimum lot sizes in SFg residential Planning 				Areas shall be 0.67 acre.

g.	Specific to PA-41 & 47, if these areas are developed for a clustered Age 				Targeted specific use than the minimum lot size may be reduced to 80 				feet in width and 120 feet in depth.  The number of units allowed in 				each planning area may not increase beyond the 19 units allowed in PA-				41, and the 18 units allowed in PA-47; however, the gross density may 				increase to a maximum of 1.2 du/ac.  The site design for these areas 				must include all homes backing to open space.

## F. 	Residential (THd)

1. 	Intent

Provide for residential Development allowing for a variety of Single Family 			Detached Dwelling Units, Zero Lot Line Dwelling Units, Single Family Attached, 			Semi-Detached, Clustered Single Family Dwelling Units, Multi-					family/Condominium Units, Duplex Dwelling Units, Townhome Dwelling Units 			and Accessory Uses.

2. 	Uses Permitted by Right

a. 	Single Family Detached Dwelling Units.

b.	Zero Lot Line Dwelling Units.

c. 	Duplex, Triplex, and Fourplex Dwelling Units.

d. 	Townhome Dwelling Units including Single Family Attached, Semi-				detached Dwelling Units and Live/Work Dwelling Units.

e. 	Clustered Single Family Dwelling Units, including but not limited to Zero 				Lot Line Dwelling Units, Patio Dwelling Units, Two, Three, and Four 				Family Dwelling Units and Atrium Dwelling Units.

f. 	Age targeted dwellings. (Not depicted on plan for this PD Submittal - 				Will be included in Site Plan/Subdivision Submittals)

g. 	Group Homes

h. 	Age restricted dwellings. (Not depicted on plan for this PD Submittal - 				Will be included in Site Plan/Subdivision Submittals)

i. 	Independent and/or Assisted Living.

j. 	A temporary sales and marketing center function developed to 					showcase a variety of builders and housing types within a limited area.

k. 	Attached or detached private garages (3 cars maximum).

I. 	Commonly associated Accessory Uses, incidental to the Principal Use or 				Building, including but not limited to: carriage house units not for sale, 				storage sheds, private greenhouses, home occupations, radio or TV 				antennas, TV satellite dishes, private tennis courts, private swimming 				pools, and private facilities for the keeping of common household pets 				according to the regulations and standards contained within the Parker 				Municipal Code.

m. 	Public Open Space.

n. 	Public and private schools and related administrative facilities for 				elementary and secondary education.

o. 	Private recreational and park uses, country clubs, recreational facilities, 				including but not limited to: tennis courts, swimming pools and jogging, 				riding, hiking and biking trails.

p. 	Public parks, playgrounds and other recreational areas.

q. 	Places of religious assembly, religious schools.

r. 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as permitted by Section 				13.04.220 of the Parker Municipal Code.

3. 	Uses Permitted by Special Review

a. 	Common recreation vehicle storage area.

b. 	Buildings, garages and utility stations related to emergency services, 				such as ambulance, fire, police and rescue.

c. 	Neighborhood public service, health and education facilities, such as 				community centers, libraries and museums.

d. 	Child Care Centers.

4. 	Accessory uses and building - Refer to Town of Parker Land Use Ordinance 				13.04.170

a. 	Shall also include utility facilities and well sites.

5. 	Development Standards

a. 	Average Density. The Average Density shall be up to ten (10) Dwelling 				Units per acre for each THd Planning Area. These average densities are 				set forth on the PD Development Plan, and reflect the number of units 				over the entire acreage for the Planning Area. Densities within portions 				of the Planning Area will vary depending on product type.

b.	Maximum Number of Dwelling Units. The maximum number of 				Dwelling Units permitted within each THd residential Planning Area shall 			not exceed 120% of the units shown on the PD Development Plan for 				each individual Planning Area.

c.	Building Setback: The minimum Building Setbacks in THd residential 				Planning Areas will vary depending upon the product type in any given 				portion of the Planning Areas. The following setbacks win be associated 				with the different product types defined as a Use Permitted by Right.

Single Family Detached Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Zero Lot Line Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Duplex Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Townhome Dwelling Unit (Including Live/Work Townhomes)

Building Front 	4 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	4 feet from corner street

Building Sides	0 feet from other property lines & lots with common 						walls

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Patio Dwelling Unit

Building Front 	8 feet (Principal Building)

10 feet (Garage perpendicular)

6 feet (Garage parallel)

Building Sides	8 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

d. 	Building Separation. The minimum building separation shall be the 				greater of:

i)	Seven and ½ (7.5) feet for Single Family Detached residential 					Buildings, ten (10) feet for Buildings containing Single Family 					Attached Dwelling Units, and twenty (20) feet for other 						Principal Buildings located within each THd Planning Area, or

ii) 	shall be governed by the applicable building code.

e. 	Building Height. No residential Buildings within THd Planning Areas shall 			exceed forty (40) feet in height except by special review.

f. 	Minimum Lot Sizes. The minimum lot sizes In THd residential Planning 				Areas shall be thirty five hundred (3500) square feet.

## G. 	Residential (THe)

1. 	Intent

Provide for residential Development allowing for a variety of Single Family 			Detached Dwelling Units, Zero Lot Line Dwelling Units, Single Family Attached, 			Semi-detached, Clustered Single Family Dwelling Units, Duplex Dwelling Units, 			Townhome Dwelling Units and Accessory Uses.

2. 	Uses Permitted by Right

a. 	All uses permitted by right within Residential (THd) Land Use Category. 				Development standards contained within the Residential (THd) Land 				Use Section shall apply, as required.

b. 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as determined by the 					Planning Director.

3. 	Uses Permitted by Special Review

a. 	All Uses permitted by Special Review within the Residential (THd) Land 				Use Category.

4. 	Accessory uses and building - Refer to Town of Parker Land Use Ordinance 			13.04.170

a. 	Shall also include utility facilities and well sites.

5. 	Development Standards

a. 	Average Density. The Average Density shall be up to fourteen (14) 				Dwelling Units per acre for each The Planning Area. These average 				densities are set forth on the PD Master Plan, and reflect the number of 				units over the entire acreage for the parcel. Densities within portions of 				the parcel will vary depending on product type.

b.	Maximum Number of Dwelling Units. The maximum number of 				Dwelling Units permitted within each THe residential Planning Area shall 			not exceed the 1.2 times the maximum number set forth on the PD 				Master Plan for each individual Planning Area.

c. 	Building Setback. The minimum Building Setbacks in THe residential 				Planning Areas will vary depending upon the product type in any given 				portion of the Planning Area. The following setbacks will be associated 				with the different product types defined as a Use Permitted by Right.

Single Family Detached Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Zero Lot Line Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Duplex Dwelling Unit

Building Front 	10 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	10 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Townhome Dwelling Unit (Including Live/Work Townhomes)

Building Front 	4 feet (Principal Building)

12 feet (Garage perpendicular)

8 feet (Garage parallel)

Building Sides	4 feet from corner street

Building Sides	0 feet from other property lines & lots with common 						walls

Building Rear	4 feet from alley

Building Rear	0 feet from other property lines

Patio Dwelling Unit

Building Front 	8 feet (Principal Building)

10 feet (Garage perpendicular)

6 feet (Garage parallel)

Building Sides	8 feet from corner street

Building Sides	0 feet from other property lines

Building Rear	4 feet from alley

d. 	Building Separation. The minimum building separation shall be the 				greater of:

i)	Seven and ½ (7.5) feet for Single Family Detached residential 					Buildings, ten (10) feet for Buildings containing Single Family 					Attached Dwelling Units, and twenty (20) feet for other 						Principal Buildings located within each THd Planning Area, or

ii) 	shall be governed by the applicable building code.

e. 	Building Height. No residential Buildings within THd Planning Areas shall 			exceed forty (40) feet in height except by special review.

f. 	Minimum Lot Sizes. The minimum lot sizes in THd residential Planning 				Areas shall be thirty five hundred (3500) square feet.   (should those be 				THe?)

## H. 	Mixed Use (MU)

1. 	Intent

To provide for the integration, horizontally and vertically, of a broad range of retail 		goods and services, business and professional services with Duplex, Triplex, Fourplex, 	Townhome, Condominiums and/or Multi-family residential development.

There are two areas of the PD Plan that are designated as MU. These MU Planning areas 	are anticipated to develop as commercial/retail centers.  Some of these 	commercial/retail centers will have a live work residential component built within these 	areas.

Should market conditions change, the developer may desire to build a residential 	project in lieu of these commercial/retail areas. By allowing both commercial and 	residential uses in the MU Planning areas, this affords the developer the opportunity to 	build what is appropriate for these areas based on market demand.

2. 	Uses Permitted by Right

a. 	Retail

(1) 	Grocery store

(2) 	Convenience retail shopping facilities, including but not limited 					to:

a. 	drug stores

b. 	liquor stores

c. 	convenience grocery

(3) 	Specialty retail including but not limited to:

a. 	art gallery

b. 	antiques

c. 	artisan shops

d. 	gift shop

e. 	pet shops

f. 	florists

g. 	book store

h. 	stationery stores

i. 	retail food specialty shops which sell food products not 						intended to be consumed on the premises

j. 	butcher shops

k.	candy stores

I. 	bakeries

m. 	doughnut shops

n. 	dairy product shops

o. 	toy and game stores

p. 	battery and accessory stores

q. 	bicycle stores

r. 	music stores

s. 	sporting goods store

(4) 	Hardware and building material stores

(5) 	General retail merchandise and apparel shops, including but not 				limited to:

a. 	junior department stores

b. 	craft and hobby stores

c. 	discount department stores

d. 	clothing stores

e. 	shoe stores

f. 	furniture stores

g. 	household appliance stores

h. 	floor covering, drapery and upholstery stores

I. 	electronics stores

j. 	cosmetic store

(6) 	Development Sales and Marketing Center

•     Eating and Drinking Establishments

(1) 	restaurants

(2) 	fast food, drive-through or carryout restaurants

(3)	ice cream parlors

(4) 	coffee shops

(5) 	delicatessens

(6) 	lounge, bar or microbrewery

•    Personal Services

(1) 	Convenience service establishments, including 							but not limited to:

a. 	barber shops and beauty salons

b. 	dry cleaners and laundries

c. 	photo studio

d. 	shoe repair shops

e. 	watch or jewelry repair

f. 	travel agency

(2) 	day care centers

(3) 	health clubs

•    Entertainment-Indoors

(1)	indoor movie theaters

(2) 	performance arts

(3) 	bowling alleys

(4) 	skating rinks (both ice and roller)

(5) 	pool halls

(6) 	arcade amusement centers

•    General Office and Professional Services

(1) 	business and professional offices

(2) 	medical and dental offices and clinics

(3) 	banks and other financial service 								establishments, with or without drive-through 							facilities

b. 	Commercial Convenience grocery.

(1) 	Convenience retail shopping facilities, including but not limited 					to: drug stores, liquor stores, florists, book and stationery 					stores, hardware stores, auto supply stores, record and tape 					stores, and retail food specialty shops which sell food products 					not intended to be consumed on the premises, such as (but not 					limited to) butcher shops, candy stores, bakeries, doughnut 					shops, dairy product shops, ice cream parlors, coffee shops and 					delicatessens.

(2) 	Convenience service establishments, including but not limited 					to: barber and beauty shops, cleaners, shoe repair shops, 					laundries, watch or jewelry repair, and travel service.

(3) 	Low intensity specialty goods and service establishments, 					including but not limited to: art gallery, antiques, artisan shops, 					photo studio, gift shop, plant store or nursery, pet shops, 					furniture store, home furnishings, newspaper office, apparel, 					and appliances.

(4) 	Conventional restaurants.

(5) 	Fast food, drive-in or carryout restaurants.

(6) 	Electronic funds transfer facilities.

(7) 	Photo service facilities.

(8) 	Medical and dental offices and clinics.

(9) 	Business and professional offices.

(10) 	Private recreational and park uses, country clubs, recreational 					facilities, including but not limited to: open space, tennis courts, 				swimming pools and jogging, riding, hiking and biking trails.

(11) 	Neighborhood public service, health and education facilities, 					such as community centers, libraries and museums, except for 					PA 3, 4, 12, 14, 35, 36, 43 and 44 which are retail uses only.

(12) 	Nursery schools and day/child care centers.

(13) 	Retail food and food service establishments, including but not 					limited to: groceries and supermarkets, and cafeterias.

(14)	General retail merchandise and apparel shops, including but not 				limited to: junior department stores; variety stores; discount 					department stores; clothing stores; accessory and specialty 					stores; shoe stores; furriers and fur stores; furniture stores; 					home furnishing stores; household appliance stores; floor 					covering, drapery and upholstery stores; and radio, television, 					stereo and video stores.

(15) 	Personal service establishments.

(16) 	General retail business establishments, including but not limited 				to: card and gift stores; tobacco shops; cosmetic store; toy and 					game stores; automotive tire, battery and accessory stores, 					garden, plant and floral shops; and bicycle stores.

(17) 	Car care centers, gasoline service/stations, tire sales and 					service, car wash.

(18) 	Commercial entertainment businesses, including but not limited 				to: indoor movie theaters, performance arts, bowling alleys, 					skating rinks (both ice and roller), pool halls, and arcade 					amusement centers.

(19) 	Banks, savings and loan and other financial service 						establishments.

(20) 	Public utility administrative and business offices.

(21) 	Buildings, garages and utility stations related to emergency 					services, such as ambulance, fire, police and rescue.

(22) 	Motels, Hotels, including detached guest units, restaurants, gift 					shops and other incidental accessory uses located within the 					principal building.

(23) 	Transit facilities.

(24) 	Universities, Colleges, or higher educational type uses.

(25) 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as determined by the 					Planning Director.

(26) 	Sales Center - conversion to a recreation facility adjacent to 					open space.

c. 	Residential

(1) 	Duplex, Triplex, and Fourplex Dwelling Units.

(2) 	Townhome Dwelling Units including Single Family Attached, 					Semi-detached Dwelling Units and Live/Work Dwelling Units.

(3) 	Condominiums

(4) 	Independent and/or Assisted Living.

(5) 	Group Homes

(6) 	Clustered Single Family Dwelling Units, including but not limited 					to Zero Lot Line Dwelling Units, Patio Dwelling Units, Two, 					Three, and Four Family Dwelling Units and Atrium Dwelling 					Units.

(7) 	A temporary sales and marketing center function developed to 					showcase a variety of builders and housing types within a 					limited area.

(8) 	Attached or detached private garages (3 cars maximum).

(9) 	Commonly associated Accessory Uses, incidental to the 						Principal Use or Building, including but not limited to: carriage 					house units not for sale, storage sheds, private greenhouses, 					home occupations, radio or TV antennas, TV satellite dishes, 					private tennis courts, private swimming pools, and private 					facilities for the keeping of common household pets according 					to the regulations and standards contained within the Parker 					Municipal Code.

(10)	Public Open Space.

(11) 	Public and private schools and related administrative facilities 					for elementary 	and secondary education.

(12) 	Private recreational and park uses, country clubs, recreational 					facilities, including but not limited to: tennis courts, swimming 					pools and jogging, riding, hiking and biking trails.

(13) 	Public parks, playgrounds and other recreational areas.

(14) 	Any other uses consistent with the purposes of this Section and 					compatible with the uses set forth herein, as permitted by 					Section 13.04.220 of the Parker Municipal Code.

(15) 	All those uses permitted within the Parks, Open Space Land Use 					categories.

3. 	Uses Permitted by Special Review

a. 	Common recreation vehicle storage area.

b. 	Buildings, garages and utility stations related to emergency services, 				such as 	ambulance, fire, police and rescue.

c. 	Neighborhood public service, health and education facilities, such as 				community centers, libraries and museums.

d. 	Child Care Centers.

e. 	Small animal veterinary clinics with no outside facilities.

4. 	Accessory uses and building - Refer to Town of Parker Land Use Ordinance 			13.04.170

a. 	Shall also include utility facilities and well sites.

5. 	Development Standards

a. 	Floor/Area Ratio. The maximum floor-area ratio for each development 				tract within the 	Mixed Use Planning Area shall be six tenths (.60) to one 				(1).

b. 	Maximum Number of Dwelling Units. The maximum density allowed 				for all Planning Areas designated as Mixed Use is 22 dwelling units per 				acre. Density adjustments and variations as described in Section I, Part 				C. cannot be used to increase the maximum allowed density within a 				Mixed Use District, unless the request is processed as a Major 					Amendment to the Development Plan and Guide.

c. 	Building Setback: Non-Residential. The minimum building setback from 				any public street right-of-way shall be:

Building front	0 feet

Building side 	0 feet

Building rear 	0 feet

d. 	Building Setback: Residential. For residential components within Mixed 				Use (MU) zoning, the building setback standards outlined In the 					Townhome/Patio Home (THe) Land Use Category shall apply.  For multi-				family Dwelling Units the setbacks shall be:

Building front	0 feet

Building side 	0 feet

Building rear 	4 feet from alley

Building rear 	0 feet

e. 	Building Separation. The minimum building or structure separation shall 			be the greater of:

i) 	ten (10) feet or

ii) 	shall be governed by the applicable building code.

f. 	Building Height. Buildings or structures in Mixed Use Planning Areas are 			permitted up to 45' in height without special review and up to 50' in 				height with Special Review. However, in Planning Areas 3, 4, 9, 12, 14, 				35, 36, 43 and 44, buildings or structures are permitted up to 50' in 				height without Special Review and up to 60' in height with Special 				Review.

## I. 	Parks

1. 	Intent

To provide for park uses which compliment residential neighborhoods. Parks to 			include pocket parks, neighborhood parks and regional parks.

2. 	Park Designations

a. 	Pocket

b. 	Neighborhood

c. 	Community

d. 	Regional ·

3. 	Development Criteria for parks

a. 	Pocket Park

(1) 	Refer to the Town of Parker Land Development Ordinance, 					Section 13.07.140 Dedication for parks, trails, open space and 					schools, specifically Section 13.07.140(c)(9) Design criteria for 					parks, trails and open space.

(2) 	Size: ¼ to ½ acre

(3) 	Location/Orientation: Centrally located within the residential 					development and/or easily accessible by residents without the 					use of vehicles. A 5-10 minute walking distance

(4) 	Frontage: Required on one, preferred two or more local streets.

(5)	Minimum development features and/or amenities: Turf and 					landscape plantings to provide shade over at least 15% of the 					area, benches in shade and sun, bike racks and trash 						receptacles. Optional features: a small shade structure, small 					playground and/or small scale tot-lot, picnic tables, minimal 					signage, security lighting.

b. 	Neighborhood Park

(1) 	Refer to Town of Parker Land Development Ordinance, Section 					13.07.140 Dedication for parks, trails, open space and schools

(2) 	Size: Determined by Town Council, refer to the Town of Parker 					Land Development Ordinance, Section 13.07.140 Dedication for 					parks, trails, open space and schools, specifically Section 					13.07.140(c)(2)(c). A minimum of 3 acres in size.

(3)	Location/Orientation: Within an immediate neighborhood with 					a ¼ to ½ mile service radius. Neighborhood parks shall serve as 					an extension of the neighborhood around them. They shall be a 					social and recreation focal point. Interconnected to 						trails/sidewalks low-volume streets within walking/biking 					distance of most users.

(4) 	Frontage: High visibility to surrounding local streets. On street 					parking

(5) 	Minimum development features and/or amenities: 50%/50% 					split between active and passive recreational uses. Active 					recreational features may include: a children's playground, 					unobstructed open play areas for practice or pick-up games and 					low impact recreation options such as bocce ball and/or 					horseshoes. Other amenities should include picnic shelters, 					internal trails, benches, trash receptacles, bike racks, public art, 					signage and security lighting.

c. 	Community Park

(1) 	Refer to Town of Parker Land Development Ordinance, Section 					13.07.140 Dedication for parks, trails, open space and schools

(2) 	Size: Determined by Town Council, refer  the Town of Parker 					Land Development Ordinance, Section 13.07.140 Dedication for 					parks, trails, open space and schools, specifically Section 					13.07.140 (c)(2)(c).  A minimum of 10 acres in size.

(3) 	Location/Orientation: Within an immediate neighborhood with 					a 2.5 mile service area. Located adjacent to open space, a 					community facility such as an activity center or a middle or 					elementary school to maximize resources and to benefit from 					joint use of such facilities as fields, multi-use courts or skate 					parks.

(4) 	Frontage: Located on arterial or collector streets and accessible 					by public transportation, personal vehicle, walking or by bicycle. 				Geographically centered within the overall development.

(5)	Development features and/or amenities: Lighted sports fields, 					drinking fountains, public art, benches with shade, picnic tables 					with shade, trash receptacles, permanent rest room facility, 					security lighting, signage, large play structures, game courts, 					disc golf, climbing wall, skate park and/or other similar popular 					activities, jogging trails, informal ball fields for youth play, low 					impact recreation options such as bocce ball and/or horseshoes, 				parking lots, an area for outdoor farmer's markets and/or 					outdoor concerts.

d. 	Regional Park

(1) 	Size: A minimum of 100 acres in size.

(2)	Location/Orientation: Located to service the community within 					a distance of a 6 mile radius, adjacent to open space, a 						community facility such as an activity center or a middle or 					elementary school to maximize resources and to benefit from 					joint use of such facilities as fields, multi-use courts or skate 					parks.

(3) 	Frontage: Located on arterial or collector streets and accessible 					by public transportation, personal vehicle, walking or by bicycle. 				Geographically centered within the overall development.

(4) 	Development features and/or amenities: A community activity 					center that includes sports fields and courts, multiuse fields that 				are both lighted and unlighted, tennis courts, basketball court, 					rest room building, concession building, play structures for the 					2-5 yrs. of age and separate play structures for 5-12 yrs. of age, 					walkways and trails, parking lots, drinking fountains, public art, 					benches with shade, picnic tables with shade, trash receptacles, 					security lighting, signage, jogging trails, low impact recreation 					options such as bocce ball and/or horseshoes.

4. 	Uses Permitted by right

a. 	Passive recreation uses and open space.

b. 	Active recreation uses, including but not limited to the following 				facilities: baseball diamonds, softball diamonds, soccer fields, tennis 				courts, volleyball courts, basketball courts, swimming pools, play 				apparatus, picnic areas, recreation center, and jogging, hiking and/or 				bicycling trails.

c. 	Free standing light fixtures 14' and under.

5. 	Accessory uses and building - Refer to Town of Parker Land Use Ordinance 			13.04.170

a. 	Shall also include utility facilities and well sites.

6. 	Development Standards

a. 	Building Setback: Street. The minimum building setback from any public 				street right-of-way line shall be:

Building front 	25 feet

Building side	20 feet

Building rear 	25 feet

b. 	Building Separation. The minimum building or structure separation shall 				be the greater of:

i) 	twenty (20) feet or

ii) 	shall be governed by the applicable building code.

c. 	Building Height. No buildings or structures shall exceed thirty-five (35) 				feet in height, except by Special Review, in which case no Buildings or 				Structures shall exceed forty-five (45) feet

## J.	Open Space (OS)

1.	Intent

To provide passive open space uses which will separate, define and protect the 			development planning areas contained within this Planned Development. Open 			Space does not include golf courses.

2. 	Uses Permitted by Right

a. 	Passive recreation uses and open space.

b. 	Jogging, hiking and/or bicycle trails.

c. 	Dog Parks

d. 	Community Gardens

e. 	Drainage Facilities

f. 	Picnic shelters.

3. 	Development Standards

a. 	Building Setback: Street. The minimum building setback from any public 				street right-of-way line shall be:

Building front 	25 feet

Building side 	15 feet

Building rear	25 feet

b. 	Building Separation. The minimum building or structure separation shall 				be the greater of:

i) 	twenty (20) feet or

ii) 	shall be governed by the applicable building code.

c. 	Building Height. No buildings or structures shall exceed thirty-five (35) 				feet in height, except by Special Review, in which case no Buildings or 				Structures shall exceed forty five (45) feet.

## K. 	Trails

The Development Plan depicts trail connections through open spaces and buffer areas and makes connections to off-site trails.

Trail connections through development parcels are anticipated. These trail connections shall be made in subsequent phases of the Town of Parker's entitlement process.

The Development Plan indicates the number of trail connections, but the exact locations  will be determined in latter phases of the entitlement process.  These trail connections 	include:

1. 	Three trail connections along the southern edge of the property (PA 22, 	23, 24, 			25 and 33).

2. 	One trail connection along the west property line.

3. 	Two trail connections from the east-west drainage in the center of the site 			through PA 29 and 31.

4. 	One trail connection from the Crowfoot Valley buffer to the MU 	Planning area 			through Planning Areas 37

5. 	Special Trail Requirements.

a. 	Future Builders of Planning Areas 22, 23, 24, 25 and 33 are advised that 				trail connections through these planning areas will be designed in 				conjunction with subdivision proposals upon these planning areas. 				These interconnections are intended to connect from the Southern 				Regional Trail along the Hess Ranch southerly boundary, internally 				through these filings, ultimately to the open space corridor located 				north of Planning Areas 16, 17, 18, 19, 20 and 32. These trail segments 				will be designed and constructed by the future buildings for Planning 				Areas 16, 17, 18, 19, 20 and 32 and secured as part of Subdivision 				Improvement Agreement for residential filings upon these specified 				Planning Areas.

Similarly, future builders of Planning Areas 16, 17, 18, 19, 20 and 32 are 				advised that each filing will be required to contribute a pro-rata share 				for the cost of constructing an internal trail within the open space 				corridor which abuts all these planning areas. This contribution will be 				determined at the time of Final Plat for each planning area and specified 			In the Subdivision Improvement Agreement for each planning area.

# IV. ADDITIONAL PLANNING AREA REQUIREMENTS

## A. 	General

As stated in Section III, Land Use Regulations, the maximum number of dwelling units permitted within each residential planning area shall not exceed 120% of the units shown on the Development Plan for each individual Planning Area. However, regardless of the unit count in each Planning Area, in no case 	wilt the total unit count of Planning Areas collectively exceed 3,379 units.

## B. 	Lone Tree Intergovernmental Agreement

At the time of the Town Council’s approval of this Development Plan and Guide, Planning Areas designated as SFe, SFf and SFg are within an area that is regulated by an Intergovernmental Agreement (IGA) between the Town of Parker and the City of Lone Tree. The number of dwelling units prescribed by the IGA is limited to 100 units for these Planning Areas. On September 5, 2015 the IGA expires. The day after, September 6, 2015 the cumulative number of dwelling units allowed for Planning Areas 27, 28, 29, 30, 31, 32 and 33 will be increased to a number greater than 100. This increase will not require a Major Amendment to the Development Plan and will be counted as part of the overall 3,379 dwelling units allowed by the Plan.

## C.	 Douglas County Intergovernmental Agreement

1. 	In the southern part of Anthology, now 'Hess Ranch' in an effort to preserve the 			rural roadside character of Crowfoot Valley Road, it will be necessary to 				maintain an average buffer area along Crowfoot Valley Road of 300', adjacent to 		Planning Areas 34 and 40, as represented on the Hess Ranch Development Plan. 			This buffer, whose intent is to screen and soften highly visible areas, should 			include enhanced landscape (greater density of planting as opposed to open 			space areas, palette of predominantly native plant materials) earth berming, 			and if necessary, open and transparent fencing, to visually minimize 				development impact. ·

2. 	The rural character of Crowfoot Valley Road shall be reflected in the 				development of this corridor, including:

a. 	The average 300' buffer between the adjacent residential development 				and Crowfoot shall be respected and the landscape in this buffer shall 				consist of earth berming, open and transparent fencing with a palette of 			plant materials that enhances and augments the existing and native 				landscape, as described in section 2, above.

b. 	Minimize the negative visual impact of parking areas through 					orientation parallel to and away from Crowfoot Valley Road, landscape 				screening and/or berming, and the presence of landscaped islands and 				planting strips within parking lots themselves to reduce the perceived 				sizes of the parking lot.

c. 	Screening of dumpster and service areas throughout the Mixed Use 				Center through landscape and/or architectural elements.

d. 	Architecture that is articulated or enhanced on those sides that face 				Crowfoot Valley Road.

# V. 	ENFORCEMENT AND ADMINISTRATION

## A. 	Incorporation of Planned Development Master Plan

The plan of development for HESS Ranch Development, including the location and boundaries of Planning Areas, the circulation elements, and the densities established by the HESS RANCH DEVELOPMENT PLAN AND GUIDE together with everything shown thereon and all amendments thereto.

## B. 	Effect of Recorded Plan

The approval by Ordinance of the HESS RANCH DEVELOPMENT PLAN AND GUIDE by the Town Council and the recording of such Development Plan and Guide with the Douglas County Clerk and Recorder shall be deemed to be rezoning to a PD-Planned Development District under Section 13.04.150 of the Parker Municipal Code for the real property described in the Development Plan.